# METHOD OF STATEMENT

Date: 07th Oct 2019

Description of work

To hoist down and unload 2 unit machine from low loader, shift and position into customer premise at ground floor

Machine Spec

2 unit Kitamura HX500i

Dim: 5.23 x 2.70 x 3.19

Wt: 16, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

11/10/19

Time of Operation

Afternoon schedule

Location of Operation

#### No 17 Kian Teck Crescent

Perfect Team Engineering Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 633 lorry loader

Heavy duty rollers

1 set wire sling

Shackle

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Hoist down machine from low loader and load onto near ground level

- Swing crane to loading area

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Shift machine into production room at ground floor

- Position machine at the designated location

- Unload machine from rollers and let it sit onto machine plate

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175