# METHOD OF STATEMENT

Date: 21 May 2015

Description of work

To unload 3 cases of Honing Equipment at total weight of 2,491 kgs from 1 x 20ft GP container, uncrate, shift and position into customer premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Delivery date: 22/05/2015

Location of Operation

#### No 17 Kian Teck Crescent

Perfect Team Engineering Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crews

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location

- Pull out machine crate from container and load onto lorry chassis

- Hoist down crate from lorry chassis and load onto ground

- Uncrate case

- After uncrate, hoist and detach machine from base

- Swing crane slowly away from base to loading area

- Lower down machine

- Insert rollers/pallet truck to machine base for the move

- Push and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175