# METHOD OF STATEMENT

Date: 14th Dec 2017

Description of work

To unload 2 unit machine from 1 x 40ft FR container, uncrate, shift and position into premise at ground level

Machine Spec

Dual Drive Press Brake BH13530 & BH8525

Dim: 450 x 228 x 340 cm / Wt: 9, 770 kgs

Dim: 410 x 210 x 321 cm / Wt: 7, 380 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

14/12/2017   –   10:00am

Location of Operation

#### No 18 Joo Koon Circle

Mazak

Equipment Use

1 x 50 ton crane

Rollers

Wire sling

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift

- Hoist up machine crate from FR container and load onto ground

- Uncrate case

- After uncrate, hoist and detach machine from base

- Swing machine to loading area

- Lower down machine slowly

- Insert rollers to machine base and secured with bolts and nuts

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175