# METHOD OF STATEMENT

Date: 17th Jan 2017

Description of work

To unload 2 unit case from lorry chassis, shift and position to temporary location without uncrating

Machine Spec

1 wooden case of 1 unit ‘KENT’ Surface Grinder

Dim: 280 x 228 x 2221 cm

Wt: 2,525 kgs

1 wooden case of I unit ‘SHM’ Cylinder Grinder

Dim: 227 x 260 x 215 cm

Wt: 2, 655 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

25/01/2017 – 10:00am

Location of Operation

#### No 18 Tractor Rd

NOK Precision Component (S) Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (certificate shall be available at all times)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, cotton gloves)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman Certificate shall be available at all times

# JOB SCOPE

- Lorry crane to park at designated location for the lift

- Hoist down machine case from lorry chassis

- Insert pallet truck

- Shift and position to temporary location without uncrating

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175