# METHOD OF STATEMENT

Date: 04 Feb 2016

Description of work

To unload 1 unit machine from low-bed, uncrate, shift and position into customer premise at ground level

Machine Spec

1 unit X-View NSI X5000-CT Digital X-Ray machine

Dim: 394 x 252 x 310 cm

Gross Wt: 20, 909 kgs

Net Wt: 16,818 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

15/02/2016 – 10:00am

Location of Operation

#### No 180 A.M.K Ave 8

Nanyang Polytechnic

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

Heavy duty rollers/pallet truck

Spreader Bar

Wire sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location

- Reverse low-bed near crane

- Hoist down crate from low-bed and load onto ground

- Uncrate case

- After uncrate, hoist and detach machine from base

- Swing crane slowly away from base to loading area

- Lower down machine

- Insert rollers to machine base for the move

- Push and position machine to the designated location inside workshop

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175