# METHOD OF STATEMENT

Date: 20th June 2019

Description of work

To shift out 1 unit machine from workshop to loading bay, using forklift to fork up machine and load onto low loader, secured machine and deliver to other location

Machine Spec

Germany ‘GROB’ 5 Axis Universal Machining Center

Dim: 580 x 302 360 cm

Wt: 13, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

25/06/2019   –   09:45 am

Location of Operation

#### No 2 A.M.K Drive, Engineering Block G-G201

ITE College Central

Equipment Use

1 x lorry crane

1 x 16 ton forklift

Rollers/pallet truck

Metal plate/canvas (for epoxy floor protection)

1 x low loader

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator will be involved in this operation

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers under machine bed

- Shift out machine from workshop to loading bay

- Using forklift to fork up machine

- Load machine onto low loader chassis

- Secured machine with strapping belt prior to moving out of premise and deliver to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175