# METHOD OF STATEMENT

Date: 27th Oct 2020

Description of work

To unload 4 cases of 1 unit machine from 1 x 40ft FR, 1 x 40ft GP and 1 x 20ft GP container, uncrate, shift and position into showroom

Machine Spec

Horizontal Y-Axis CNC Lathe Integrex I-450H

Case 1: 6.5m x 3.1m x 3.36m / Wt: 17, 550 kg

Case 2: 5.3m x 2.29m x 1.09m / Wt: 1, 070 kg

Case 3: 1.73m x 1.71m x 1.54m / Wt: 900 kg

Case 4: 5.3m x 1.09m x 2.07m / Wt: 1, 210 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

30/10/2020

Time of Operation

09:45 am

Location of Operation

#### No 21 Joo Koon Circle

Yamazaki Mazak Singapore Pte Ltd

Equipment Use

1 x 50ton mobile crane

1 x 633 lorry loader

Wire/webbing sling

1 x low-bed

Factory 20ton overhead crane

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated location for the hoisting

- Hoist down case from FR container to the ground

- Uncrate case

- Hoist and detach machine from base

- Load machine onto low-bed chassis

- Reverse low-bed to inside factory unloading bay

- Using customer overhead crane to hoist up machine from low-bed chassis

- Shift and position machine to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175