# METHOD OF STATEMENT

Date: 04th May 2017

Description of work

1) To unload 4 cases of 1 unit Horizontal Machining Center and 7 cases of 1 unit Multiple Pallet Stoker from 2 x 40ft FR, 1 x 20ft GP and 1 x 40ft GP container and put aside

2) To unload 5 cases of Integrex I-2005 from 1 x 40ft FR, 1 x 20ft GP and 1 x 40ft HQ container, uncrate, shift and position into showroom

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

05/05/17   -   Unload without uncrating

09/05/17   -   Unload, uncrate, shift and position into showroom

Location of Operation

#### No 21 Joo Koon Circle

Yamazaki Mazak Singapore Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 x lorry crane

1 x forklift

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Hoist up machine case from FR container and load onto ground

- Uncrate case

- After uncrate, hoist up machine and load onto low-bed

- Reverse low-bed to inside factory

- Using customer overhead crane to unload machine from low-bed

- Shift and position machine to location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175