# METHOD OF STATEMENT

Date: 11th Oct 2016

Description of work

To unload 4 cases of 1 unit Mazak Horizontal NC Lathe from 1 x 40ft GP container, uncrate, shift and position into premise

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

14/10/2016 – 10:00am

Location of Operation

#### No 21 Joo Koon Circle

Yamazaki Mazak Singapore Pte Ltd

Equipment Use

1 x lorry crane

Mazak overhead crane

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift (back to back with container)

- Level up lorry crane with container

- Pull out machine from container and load onto lorry chassis

- Uncrate case

- Transport machine into showroom loading bay

- Using overhead crane to hoist machine from lorry chassis and position to location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175