# METHOD OF STATEMENT

Date: 28th Nov 2022

Description of work

To unload 4 cases of 1 unit machine from lorry chassis, uncrate, shift and position into showroom

Machine Spec

Mazak VCN-430A/L

Case 1: 3140 x 2270 x 2540 mm / Wt: 5, 500 kg

Case 2: 3000 x 1750 x 850 mm / Wt: 690 kg

Case 3: 3400 x 900 x 1600 mm / Wt: 460 kg

Case 4: 1300 x 1000 x 1850 mm / Wt: 390 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

30/11/22

Time of Operation

09:45 am

Location of Operation

#### No 21 Joo Koon Circle

Yamazaki Mazak Singapore Pte Ltd

Equipment Use

3 x 633 lorry loader

Wire/webbing sling

Factory 20ton overhead crane

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for the hoisting

- Hoist down case from lorry chassis  to the ground

- Uncrate case

- Hoist and detach machine from base

- Load machine onto lorry chassis

- Reverse lorry to inside factory unloading bay

- Using customer overhead crane to hoist up machine from lorry chassis

- Shift and position machine to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175