# METHOD OF STATEMENT

Date: 27th Dec 2016

Description of work

To unload 1 unit Variaxis i-700 Vertical Machining Center and 2 unit HCN-5000 Horizontal Machining Center from 1 x 20ft FR, 3 x 40ft FR, 1 x 40ft GP and 2 x 20ft GP container, uncrate, shift and position into premise

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

28/12/2016 – 10:00am

Location of Operation

#### No 21 Joo Koon Circle

Yamazaki Mazak Singapore Pte Ltd

Equipment Use

1 x 50 ton lorry crane

Lorry crane

Heavy duty rollers

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift

- Hoist up machine case from FR container and load onto ground

- Uncrate case

- After uncrate, hoist up machine onto lorry chassis

- Reverse lorry to inside factory

- Using customer overhead crane to unload machine from lorry chassis

- Shift and position machine to location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175