# METHOD OF STATEMENT

Date: 04th May 2017

Description of work

To unload 10 steel cases of 5 unit machine from 1 x 20ft FR, 2 x 40ft FR, 1 x 40ft GP and 1 x 20ft GP container, uncrate, shift and position into customer premise at ground floor

Machine Spec

Horizontal Machining Center machine

Dim: 485 x 239 x 299 cm

GW: 11, 300 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

06/05/2017 – 09:30am

Location of Operation

#### No 23 Changi North Crescent

UMS

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

Heavy duty rollers/pallet truck

Manual/hydraulic jack

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Unload machine case from container onto ground

- Uncrate case

- Hoist and detach machine from base

- Swing machine to loading area

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Shift and position machine to the designated location at ground floor

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remarks

MOS subject to change by supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175