# METHOD OF STATEMENT

Date: 21st Oct 2016

Description of work

To unload 4 cases of 1 unit machine from 1 x 40ft FR and 1 x 20 GP container, uncrate, shift and position into customer premise at ground floor

Machine spec

Mazak MTN 1600M CNC

Dim: 4960 x 3560 x 3800 mm

Wt: 25, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Est: 12/11/2016   –   09:30am

Location of Operation

#### No 23 Loyang Way

GE Aviation

Equipment Use

1 x 70 ton mobile crane

1 x 50 ton lorry crane

Heavy duty rollers

Wire/webbing sling

1 x Forklift

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Hoist up machine case from container and load onto ground

- Uncrate case

- Hoist and detach machine from base

- Swing machine to loading area

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Shift and position machine to the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175