# METHOD OF STATEMENT

Date: 23rd May 2018

Description of work

To unload 1 unit machine from lorry chassis, uncrate, shift and position into customer premise at ground floor

Machine Spec

Hwacheon CNC Lathe / M: Hi-Tech 230BL YMC

Dim: 488 x 220 x 250 cm

Wt: 7, 560 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

28/05/2018   –   09:45 am

Location of Operation

#### No 249 Jalan Boon Lay

ST Kinetic

Equipment Use

1-2 x lorry crane

Heavy duty rollers/pallet truck

Wire/Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift

- Hoist down case from lorry chassis and load onto ground

- Uncrate case

- Hoist up and detach machine from base

- Swing machine to loading area

- Insert rollers to machine base and secured with bolts and nuts

- Transport machine to inside factory

- Shift and position machine to the designated location

- Unload machine from rollers and let it sit onto machine plate

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

REMARK:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175