# METHOD OF STATEMENT

Date: 02nd Aug 2018

Description of work

To unload 1 unit machine from lorry chassis, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 unit Harich LZ 280G Lathe

Dim: 210 x 140 x 211 cm

Wt: 1, 550 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

TBA

Location of Operation

#### No 249 Jalan Boon Lay

Advanced Material Engineering Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Lower down machine to near ground level

- Insert roller/pallet truck onto machine base

- Transport machine to inside factory

- Shift and position machine to the designated location

- Unload machine from rollers/pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

REMARK:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175