# METHOD OF STATEMENT

Date: 19th June 2020

Description of work

To unload 3 cases of 1 unit machine from 1 x 40ft FR container to the ground, uncrate, shift and position into customer premise at ground floor

Machine Spec

EMCO CNC Turning Hypertum 45

Case 1: 363 x 245 x 240 cm / Wt: 4, 680 kg

Case 2: 353 x 210 x 200 cm / Wt: 1, 380 kg

Case 3: 230 x 225 x 235 cm / Wt: 1, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

03/03/2020

Time of Operation

10:00 am

Location of Operation

#### 249 Jalan Boon Lay

Advanced Material Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader – PALFINGER PK 100002F

Rollers/pallet truck

1 set wire sling

6.5/8.5 ton shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area

- Hoist up machine case from FR container and load onto ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Lower down machine to near ground level

- Insert rollers to machine base and secured with bolts and nuts

- Shift and transport machine into inside factory

- Position machine at the designated location

- Unload machine from rollers and let it sit onto machine plate

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175