# METHOD OF STATEMENT

Date: 28th Feb 2020

Description of work

To unload 2 unit case from low-bed to the ground, uncrate, shift and position into customer premise at ground floor

Machine Spec

YCM Vertical Machining Center / TV146B

Case 1: 4.05 x 3.48 x 3.47 m

Wt: 12, 200 kgs

Case 2: 0.86 x 1.06 x 0.75 m

Wt: 460 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

03/03/2020

Time of Operation

10:00 am

Location of Operation

#### 249 Jalan Boon Lay

Advanced Material Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x low-bed

Heavy duty rollers

1 set wire sling

Shackle

Canvas/metal sheet (for epoxy floor protection)

1 x battery forklift

1 x scissor lifter

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting location

- Reverse low-bed to near crane

- Hoist up machine case from low-bed and load onto ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Lower down machine to near ground level

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Shift and transport machine to inside factory

- Position machine at the designated location

- Unload machine from rollers and let it sit onto machine plate

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175