# METHOD OF STATEMENT

Date: 19th Nov 2019

Description of work

To unload 3 cases of 1 unit machine from 1 x 40ft FR container to the ground, uncrate, shift and position into inside factory at ground floor

Machine Spec

1 unit CNC Lathe

Case 1: 365 x 247 x 242 cm   /   Wt: 4, 920 kgs

Case 2: 260 x 228 x 205 cm   /   Wt: 1, 400 kgs

Case 3: 225 x 230 x 240 cm   /   Wt: 1, 140 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

TBA

Location of Operation

#### No 249 Jalan Boon Lay

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for the lift

- Hoist down cases from container to the ground

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Lower down machine to near ground level

- Insert roller/pallet truck onto machine base

- Shift and transport machine to inside factory

- Position machine at the designated location

- Unload machine from rollers/pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175