# METHOD OF STATEMENT

Date: 13th Oct 2016

Description of work

To unload 1 unit machine with accessories from 1 x 40ft FR and 1 x 40ft OT container, uncrate, shift and position into customer production area

Machine Spec

1 unit Boring Machine Model: HB1620-130H

Case 1: 440 x 310 x 230 cm / GW: 11,500 kgs

Case 2: 668 x 295 x 230 cm / GW: 16,300 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

18/10/2016 – 10:00am

Location of Operation

#### No 25 Pioneer Crescent

Bohler Pacific Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

Heavy duty rollers

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Hoist up machine case from container and load onto ground

- Uncrate case

- Hoist and detach machine from base

- Swing machine to loading area

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Shift and position machine to the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175