# METHOD OF STATEMENT

Date: 21 March 2016

Description of work

To unload 2 unit machine from 1 x 40ft container, uncrate, shift and position into customer production area at ground floor

Machine Spec

Mazak QTN-350 CNC

Dim : 358 x 205 x 200 cm

Wt: 7,200 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

23/03/2016 – 10:00 am

Location of Operation

#### No 273 Jalan Ahmad Ibrahim

Baker Hughes

Equipment Use

1 x 50 ton lorry crane

1 x lorry crane

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location (back to back with container)

- Pull out machine from container and load onto lorry chassis

- Hoist down machine case to the ground

- Uncrate case

- Hoist and detach machine from base

- Swing crane slowly away from base to loading area

- Lower down machine

- Insert rollers to machine base

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175