# METHOD OF STATEMENT

Date: 29th July 2020

Description of work

To unload 10 cases of 1 unit from 1 x 20ft FR and 3 x 40ft HQ container, uncrate, shift and position into customer premise at ground floor

Machine Spec

Kuraki KBT 11WA

Case 1: 480 x 271 x 131 cm / Wt: 9, 200 kg

Case 2: 352 x 142 x 190 cm / 3, 800 kg

Case 3: 450 x 226 x 126 cm / 7, 300 kg

Case 4: 307 x 105 x 167 cm / 2, 400 kg

Case 5: 224 x 164 x 258 cm / 940 kg

Case 6: 311 x 187 x 196 cm / 1, 360 kg

Case 7: 345 x 127 x 134 cm / 1, 390 kg

Case 8: 80 x 70 x 162 cm / 2, 190 kg

Case 9: 417 x 227 x 132 cm / 1, 900 kg

Case 10: 482 x 222 x 187 cm / 2, 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

05/08/2020   -    09:45 am

Location of Operation

#### 3 Joo Koon Circle

AME International Pte Ltd

Equipment Use

2 x 633 lorry loader

Heavy duty rollers

1 set wire

Webbing sling

Shackle

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Insert wire sling under machine case

- Hoist down machine case from FR container to the ground

- Uncrate machine case

- Hoist up machine and load onto lorry chassis

- Transport lorry crane with machine into inside factory

- Using customer overhead crane to unload machine

- Position machine at the designated location

- Lower down and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175