# METHOD OF STATEMENT

Date: 11th Feb 2020

Description of work

To unload 10 steel cases of 1 unit Horizontal Machining Center from 1 x 40ft FR, 1 x 20ft FR and 3 x 40ft HQ container, uncrate, shift and position into customer premise at ground floor

To assist AME engineer in installation of machine

Machine Spec

Horizontal Machining Center / M: KBN 135C

Case 1: 6500 x 2600 x 2000 mm / Wt: 20, 600 kg

Case 2: 3050 x 2240 x 1690 mm / Wt: 7, 450 kg

Case 3: 4450 x 3000 x 2000 mm / Wt: 10, 000 kg

Case 4: 4800 x 2200 x 1500 mm / Wt: 8, 000 kg

Case 5: 4450 x 2240 x 2000 mm / Wt: 1, 910 kg

Case 6: 3980 x 2240 x 2090 mm / Wt: 1, 500 kg

Case 7: 3480 x 2240 x 1790 mm / Wt: 1, 500 kg

Case 8: 4400 x 2240 x 2090 mm / Wt: 1, 690 kg

Case 9: 5100 x 1925 x 1690 mm / Wt: 8, 860 kg

Case 10: 5000 x 2240 x 2000 mm / Wt: 1, 880 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

13-15/02/2020   -    09:45 am

Location of Operation

#### 3 Joo Koon Circle

AME International Pte Ltd

Equipment Use

1 x 50 ton mobile crane

2 x 633 lorry loader

Heavy duty rollers

1 set wire/webbing sling

Shackle

Metal sheet/canvas (for floor protection)

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated location

- Insert wire sling under machine case

- Hoist down machine case from FR container to the ground

- Uncrate machine case

- After uncrate, hoist up machine and detach from base

- Swing crane to unloading area

- Lower down machine slowly to near ground level

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Transport machine into production area at ground floor

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175