# METHOD OF STATEMENT

Date: 26 May 2015

Description of work

To unload 7 unit of Okuma CNC Vertical Machining Center machine from container, uncrate, shift and position into customer premise at ground level

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Est Delivery Date: 02/06/2015 – 10:00am

Location of Operation

#### No 31 Joo Koon Circle

Flextronics Manufacturing Singapore

Equipment Use

2 x lorry crane

Heavy duty rollers/air rollers

Wire/webbing sling

1 x Forklift

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift (back to back with container)

- Barricade operation area

- Pull out machine crate from container onto lorry chassis

- Hoist down machine crate from lorry chassis and load onto ground

- Uncrate case

- After uncrate, hoist and detach machine from base

- Load machine onto lorry chassis

- Move lorry to main entrance

- Hoist down machine to the ground

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Push machine to about 3 to 4m inside factory

- Jack up machine 4” high and replace heavy duty rollers to air rollers (to prevent damage to epoxy floor)

- Push and position machine to the designated area

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175