# METHOD OF STATEMENT

Date: 18th May 2016

Description of work

To unload 3 package of 1 unit Bystronic CNC Press Break machine from 1 x 20ft open top container, uncrate, shift and position into customer premise at ground level

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Estimate: 31/05/2016 - 10:00am

Location of Operation

#### No 33 Tuas Ave 2

ST Kinetics Integrated Engineering Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

Rollers/pallet truck

Wire sling/shackle

1 x Forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Open up container canvas using ladder

- Secure wire sling to machine hoisting point

- Hoist up machine from container and lower down to ground level

- Insert rollers to machine base and secured with bolts and nuts

- Push and position machine to the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175