# METHOD OF STATEMENT

Date: 11th June 2019

Description of work

To unload 2 cases of 1 unit machine from 1 x 40ft FR and 1 x 40ft GP container, uncrate, shift and position into customer premise at ground floor

Machine Spec

Hi-Tech 850BBXL CNC Lathe

Case 1: 9800 x 3540 x 3570 mm

N/G Wt: 33, 700 kgs / 37, 400 kgs

Case 2: 9800 x 2250 x 1800 mm

N/G Wt: 2, 000 kgs / 2, 330 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

15/06/2019

Time of Operation

09:45 am

Location of Operation

#### No 37 Joo Koon Rd

Energy Pipeline Services Pte Ltd

Equipment Use

1 x 100 ton mobile crane

2 x lorry crane

Heavy duty rollers

1 set wire/webbing sling

Shackle

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of 100 ton mobile crane at designated hosting location

- Insert wire sling under machine case

- Hoist down case from FR container to the ground

- Uncrate machine case

- Hoist up machine and detach from base

- Swing machine away from base

- Lower down machine slowly to near ground level

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Transport machine into production area

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd / Tel: 62824175