# METHOD OF STATEMENT

Date: 25th Oct 2017

Description of work

To unload 1 wooden case at loading bay, uncrate, hoist and detach machine from base, shift and position into customer premise at ground floor

Machine Spec

1 set Manual 4 Tank Ultrasonic Cleaner

Dim: 578 x 215 x 181 cm

Net Wt: 1, 200 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Est: 30/10/2017   –   09:30am

Location of Operation

#### 39 Changi North Crescent

Goodrich Aerospace Pte Ltd (UTC)

Equipment Use

1 x lorry crane

Pallet truck/rollers

Webbing/wire sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated loading area for unloading

- Unload case from lorry chassis

- Uncrate machine case

- Hoist up and detach machine from base

- Swing machine to loading area

- Insert pallet truck/rollers to machine base

- Shift machine into premise and position to the designated location at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175