# METHOD OF STATEMENT

Date: 11th Apr 2017

Description of work

To unload/hoist down 1 unit machine from lorry chassis, shift and position into customer premise at ground level

Machine Spec

1 unit Campro CNV-1050

Dim: 3000 x 2200 x 2400 mm

Wt: 6, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

12/04/2017 – 10:00am

Location of Operation

#### No 42B Penjuru Rd

Mencast Offshore & Marine Pte Ltd

Equipment Use

1 x 50 ton lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris, if any will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift

- Hoist up and detach machine from base

- Lower down machine to near ground level

- Swing machine to loading area

- Insert rollers to machine base and secured with bolts and nuts

- Shift and position machine to the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175