# METHOD OF STATEMENT

Date: 09 September 2015

Description of work

Using forklift to unload 1 unit machine from lorry chassis to loading bay, shift and position into customer premise at ground floor

Machine Spec

1 x Goodway GA2000 CNC

Dim: 270 x 163 x 173 cm

Wt: 3500 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

16/09/2015 – 13:00 pm

Location of Operation

#### No 5 Bedok South Rd

Yokogawa Engineering Asia Pte Ltd

Equipment Use

2 x lorry crane

Rollers/pallet truck

Wire/webbing sling

4.5 ton forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry crane operator and forklift operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- 4.5 ton forklift to fork down machine from lorry chassis

- Transport machine to loading bay

- Lower down machine onto woodblock

- Insert rollers/pallet truck to machine base

- Shift and position machine to the designated location inside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175