# METHOD OF STATEMENT

Date: 09 September 2015

Description of work

To hoist and unload 1 unit machine from lorry chassis, shift and position into customer premise at ground floor

Machine Spec

1 x Goodway GA2000 CNC

Dim: 270 x 163 x 173 cm

Wt: 3500 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

16/09/2015 – 13:00 pm

Location of Operation

#### No 5 Bedok South Rd

Yokogawa Electric Asia Pte Ltd

Equipment Use

2 x lorry crane

Rollers/pallet truck

Wire/webbing sling

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift

- Hoist and unload machine from lorry chassis to the ground

- Hoist up and detach machine from base

- Swing crane slowly away from base to loading area

- Lower down machine

- Insert rollers to machine base

- Use of battery forklift to assist in pulling machine up slope

- Shift and position machine to the designated location inside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175