# METHOD OF STATEMENT

Date: 07 Jan 2016

Description of work

To unload 1 unit Autoclave machine from 1 x 20ft OT container, shift and position into customer premise at ground level

Machine Spec

1 x Autoclave Machine

Dim: 7800 x 4750 x 4000

Wt: 33,000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

09/01/2016 – 10:00 am

Location of Operation

#### Aerospace Lab 50 Nanyang Ave

School of Mechanical & Aerospace Engineering N3.2.B4-01

Equipment Use

1 x 160 ton mobile crane

Lorry crane

1 x low loader

Heavy duty rollers

Wire sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Assignment of traffic controller to assist in controlling the traffic flow

Cordoned off work area to prevent of unauthorized entry                                                                                                                           All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Reverse container near crane

- Hoist up machine from container

- Drive container away

- Lower down machine to the ground

- Swing crane slowly to loading area

- Insert heavy duty rollers to machine base

- Shift and position machine to the designated location inside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175