# METHOD OF STATEMENT

Date: 21 August 2015

Description of work

To unload 4 cases of 2 unit machine from lorry chassis, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 x RXP 500

Dim: 3200 x 3230 x 2500 mm

Wt: 6000 kgs

1 x RXP 801

Dim: 3400 x 3230 x 2540 mm

Wt: 8600 ks

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Est: 07/09/2015 – 10:00 am

Location of Operation

#### No 51 Joo Koon Circle

Omni Mold Limited

Equipment Use

Lorry crane

Heavy duty rollers/pallet truck

Wire sling

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift

- Hoist and unload cases from lorry chassis to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane slowly away from base to loading area

- Lower down machine

- Insert rollers to machine base

- Shift and position machine to the designated location inside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175