# METHOD OF STATEMENT

Date: 02nd Sep 2019

Description of work

To unload 8 cases of 2 sets machine from 1 x 40ft FR and 1 x 40ft HQ container, uncrate, shift and position into customer premise at ground floor

Machine Spec

Agie Charmiles Cut 200 CNC Wire Cut

2 cases dim: 280 x 232 x 256 cm / Wt: 4, 550 kgs each

2 cases dim: 187 x 242 x 202 cm / Wt: 875 kgs each

2 cases dim: 143 x 130 x 159 cm / Wt: 298 kgs each

2 cases dim: 120 x 95 x 95 cm / Wt: 600 kgs each

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

09/09/2019   –   09:30am

Location of Operation

#### No 51 Seletar Aerospace View

Pratt & Whitney NGPF Manufacturing Company

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

1 x 10 ton forklift

1 x 3 ton battery forklift

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane and forklift operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- 10 ton forklift to fork down cases from container

- Unload to the ground

- Uncrate case

- Fork up and detach machine from base

- Transport machine to entrance door

- Lower down machine onto wooden block

- Insert rollers/pallet truck to machine base

- Shift machine into production area

- Position machine to the designated location at ground floor

- Unload machine from rollers/pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175