# METHOD OF STATEMENT

Date: 09th July 2018

Description of work

To unload 16 cases of 4 sets machine from 1 x 40ft FR, 1 x 40ft GP and 1 x 20ft GP container, uncrate, shift and position into customer premise at ground floor

Machine Spec

Agie Charmiles Cut 200 CNC Wire Cut

Case 1: 280 x 232 x 2536 cm / Wt: 4, 550 kgs

Case 2: 242 x 187 x 202 cm / Wt: 875 kgs

Case 3: 143 x 130 x 159 cm / Wt: 298 kgs

Case 4: 120 x 95 x 95 cm / Wt: 600 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

17/07/2018   –   09:30am

Location of Operation

#### No 51 Seletar Aerospace View

Pratt & Whitney NGPF Manufacturing Company

Equipment Use

1 x 50 ton lorry crane

Rollers/pallet truck

Wire/webbing sling

1 x 10 ton forklift

1 x 3 ton battery forklift

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane and forklift operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- 10 ton forklift to unload cases from container

- Uncrate case

- Fork up and detach machine from base

- Transport machine to entrance door

- Lower down machine onto wooden block

- Insert rollers/pallet truck to machine base

- Shift machine into production area

- Position machine to the designated location at ground floor

- Unload machine from rollers/pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175