# METHOD OF STATEMENT

Date: 18th Dec 2017

Description of work

To unload 16 cases of 4 sets machine from 4 x 40ft FR container, uncrate, shift and position into customer premise at ground level

Machine Spec

Agie Charmiles Cut 200 CNC Wire Cut

Case 1: 281 x 232 x 254 cm / Wt: 4, 472 kgs

Case 2: 184 x 242 x 202 cm / Wt: 875 kgs

Case 3: 143 x 130 x 159 cm / Wt: 298 kgs

Case 4: 113 x 93 x 95 cm / Wt: 610 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

22/12/2017   –   09:30am

Location of Operation

#### No 51 Seletar Aerospace View

Pratt & Whitney NGPF Manufacturing Company

Equipment Use

1 x 50 ton lorry crane

Lorry crane

Heavy duty rollers

Wire/webbing sling

1 x 4.5 ton forklift

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of crane at designated location for the lift

- Hoist up machine case from FR container and load onto ground

- Uncrate case

- After uncrate, hoist and detach machine from base

- Swing machine to loading area

- Lower down machine slowly

- Insert rollers/pallet truck to machine base

- Shift and position machine to the designated location at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175