# METHOD OF STATEMENT

Date: 13th Sep 2018

Description of work

To unload 3 cases of 1 unit machine from 1 x 40ft HQ and 2 x 20ft FR container, uncrate, shift and position into customer premise at ground floor

Machine Spec

UNIKA Milling Machine

Case 1: 550 x 300 x 327 cm   /   G/Wt: 19, 200 kgs

Case 2: 525 x 235 x 284 cm   /   G/Wt: 2, 720 kgs

Case 3: 208 x 158 x 240 cm   /   G/Wt: 1, 460 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Est: 25/09/2018   –   09:30am

Location of Operation

#### No 51 Seletar Aerospace View

Pratt & Whitney NGPF Manufacturing Company

Equipment Use

1 x 70 ton mobile crane

Lorry crane

Heavy duty rollers

Wire sling

Shackle

Canvas/metal sheet (for epoxy floor protection)

1 x battery forklift

1 x scissor lift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Hoist up machine case from FR container and load onto ground

- Uncrate case

- Hoist up and detach machine from base

- Swing machine to loading area

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Transport machine into inside factory

- Shift and position machine at the designated location

- Unload machine from rollers and let it sit onto machine plate

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

REMARK:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175