# METHOD OF STATEMENT

Date: 06th Nov 2019

Description of work

To unload 2 cases of 1 unit machine from 2 x 20ft FR container, uncrate, shift and position into customer premise at ground floor

Machine Spec

UNIKA Milling machine

Dim: 546 x 300 x 328 cm

Wt: 21, 800 kg

Dim: 525 x 235 x 284 cm

Wt: 2, 900 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est: 18/11/2019

Time of Operation

09:30 am

Location of Operation

#### No 51 Seletar Aerospace View

Pratt & Whitney NGPF Manufacturing Company Pte Ltd

Equipment Use

1 x 70 ton mobile crane

Lorry crane

Heavy duty rollers

1 set wire/webbing sling

Shackle

Metal sheet/canvas (for epoxy floor protection)

1 x 3 ton battery forklift

1 x scissor lift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location

- Insert wire sling under machine case

- Hoist up machine case to check on balancing

- When well balancing, hoist down machine case from FR container

- Unload case to the ground

- Uncrate case with the use of scissor lifter

- After uncrate, hoist machine and detach from base

- Shift machine base away using forklift

- Lower down machine slowly to near ground level

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Transport machine into production area

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175