# METHOD OF STATEMENT

Date: 27 Oct 2014

Description of work

To unload 2 unit machine from 2 x 20ft and 1 x 40ft FR container, uncrate, shift and position into customer premise at ground level

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Est delivery date: 03/11/2014 – 09:30am

Location of Operation

#### No 51 Seletar Aerospace View

Pratt & Whitney

- Equipment Use

1 x 70 ton mobile crane

Lorry crane

Heavy duty rollers/air rollers

Wire sling

1 x Forklift

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

- Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

- Lifting Crews

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M cert.

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Barricade operation area

- Hoist up machine crate from FR container and load onto ground

- Uncrate case

- After uncrate, hoist and detach machine from base

- Swing machine slowly to loading area

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Push machine to about 3 to 4m inside factory

- Jack up machine 4” high and replace heavy duty rollers to air rollers (to prevent damage to epoxy floor)

- Push and position machine to the designated area

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175