# METHOD OF STATEMENT

Date: 15 April 2016

Description of work

To unload 3 cases of 1 unit machine from lorry chassis, uncrate, shift and position into customer premise

Machine Spec

1 unit Mitsui MSG-300PC-NC

Dim: 270 x 200 x 221 cm

Wt: 4,400 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

19/04/2016 – 10:00am

Location of Operation

#### No 535 Yishun Industrial Pk A

ASM Technology Singapore

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for the lift

- Uncrate machine case

- Hoist and detach machine from base

- Swing machine to loading area

- Insert rollers/pallet truck to machine base

- Push and position machine to the designated location

- Lift up machine, pull out rollers/pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175