# METHOD OF STATEMENT

Date: 16th Jan 2019

Description of work

To unload 1 unit VS 97 Milling machine at unloading bay using lorry crane, shift and position to designated location

Risk Assessment

A copy of Risk Assessment will be attached

Date/Time of Operation

17/01/2019   -   10:00 AM

Location of Operation

#### No 60 Tuas Ave 11

Panduit Singapore Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Wire sling

Shackle

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry crane at unloading bay

- Using crane to lift up machine

- Insert pallet truck to machine base

- Lower down machine and let it sit on the pallet truck

- Transport machine into premise

- Shift and position machine at the designated location

- Unload machine from pallet truck and insert pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175