# METHOD OF STATEMENT

Date: 25th July 2017

Description of work

To unload 1 unit machine from lorry chassis, shift and position into customer premise at ground floor

Machine Spec

1 unit Okamoto / Model: Linear 350B

Dim: 1727 x 1526 x 1405 mm

Wt: 940 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate: 26-29/07/2017

Location of Operation

#### No 60 Tuas Ave 11

Panduit Singapore Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location

- Hoist down machine from lorry chassis

- Swing crane slowly to loading area

- Insert pallet truck to machine base

- Shift and position machine to the designated location at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175