# METHOD OF STATEMENT

Date: 27 Oct 2015

Description of work

To unload 1 unit Special Key Way CNC machine from 1 x 40ft HQ and 1 x 20ft GP container, uncrate, shift and position into customer premise at ground floor

Machine Spec

Case 1 Dim: 1180 x 200 x 243 cm

Wt: 21,300 kg

Case 2 Dim: 200 x 190 x 200 cm

Wt: 1,150 kg

Case 3 Dim: 227 x 182 x 200 cm

Wt: 1,800 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

To be advised

Location of Operation

#### No 7 Benoi Crescent

Reda Production Systems

Equipment Use

1 x 50 ton mobile crane

Lorry crane

Rollers/pallet truck

1 x forklift

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location (back to back with container)

- Pull out machine crate from container and load onto lorry chassis

- 50 ton mobile crane to hoist down machine case from lorry chassis and load onto ground

- Uncrate case

- Hoist and detach machine from base

- Swing crane slowly to loading area

- Lower down machine

- Insert rollers to machine base for the move

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175