# METHOD OF STATEMENT

Date: 10th Nov 2016

Description of work

To unload 1 set of Conveyor line from 1 x 40ft container, uncrate, shift and position into production area

Machine Spec

Refer to attached packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

To be advised

Location of Operation

#### No 7 Benoi Crescent

Reda Production Systems

Equipment Use

1 x lorry crane

Rollers/pallet truck

1 x 3 ton battery forklift

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, forklift operator and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location (back to back with container)

- Pull out machine case from container onto lorry chassis

- Forklift to unload case from lorry chassis

- Uncrate case

- Fork up and detach conveyor from case

- Transport to loading door entrance

- Lower down conveyor and insert pallet truck to conveyor base

- Shift and position conveyor to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175