# METHOD OF STATEMENT

Date: 23 April 2015

Description of Work

To hoist up 1 unit machine to 7th floor

Machine Spec

1 unit Hurco CM 6 CNC Lathe

Dim: 2.4 x 1.7 x 1.7

Wt: 3200 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

To be advised (prior to approval from management)

Location of Operation

#### No 7 Gambas Crescent #07-12

- Equipment Use

70 ton mobile crane

Wire/webbing sling

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

- Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

- Lifting Crews

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc...) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Barricade operation area

- Hoist up machine to 7th floor opening

- Lower down machine slowly

- Insert rollers/pallet truck to machine base

- Shift and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175