# METHOD OF STATEMENT

Date: 08th Dec 2017

Description of work

To unload 11 packages of 1 unit Fiber Optic Laser Cutting machine from 1 x 40ft HQ and 1 x 40ft OT container, transfer main machine onto low bed, transport to 5th floor via ramp up and using lorry crane to hoist down and machine position to location

Machine Spec

1 unit Fiber Laser Cutting

Dim: 5700 x 4680 x 2360 mm

Wt: 11, 005 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

15/12/2017   -   09:45 am

Location of Operation

#### No 71 Woodlands Ave 10 #05-13

Tech VJ Engineering Pte Ltd

Equipment Use

1 x 50 ton lorry crane

1 x lorry crane

1 x low loader

Wire/webbing sling

Rollers/pallet truck

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, mobile/lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- 50 ton lorry crane to park designated location

- Hoist up machine from OT container, load to low loader and secured with strapping belt

- Low loader to transport machine to 5th floor via ramp up

- Using lorry crane to hoist down machine from low loader

- Insert rollers to machine base

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175