# METHOD OF STATEMENT

Date: 06th Jan 2017

Description of work

To unload 1 unit machine from low loader, uncrate, shift and position into customer premise at ground level

Machine Spec

1 unit CMM machine

Dim: 280 x 300 x 320 cm

Wt: 6,300 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

09/01/2017 – 10:00am

Location of Operation

#### No 73 Nanyang Drive

Equipment Use

1 x 50 ton lorry crane

Heavy duty rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift

- Hoist down machine crate from low loader and load onto ground

- Uncrate case

- Hoist and detach machine from base

- Swing crane to loading area

- Insert rollers/pallet truck to machine base

- Shift and position machine to the designated location inside workshop

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175