# METHOD OF STATEMENT

Date: 02 June 2015

Description of work

To unload 1 unit machine from container, uncrate, shift and position into customer premise at ground level

Machine Spec

1 unit DMG CNC Turning

Dim: 3415 x 2106 x 2200 mm

Wt: 6,040 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

05/06/2015 – 10:00am

Location of Operation

#### No 73 Nanyang Drive-Valley Blk V2-E-07

Equipment Use

1 x lorry crane

Heavy duty rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location for the lift (back to back with container)

- Pull out machine crate from container onto lorry chassis

- Hoist down machine crate from lorry chassis and load onto ground

- Uncrate case

- After uncrate, hoist and detach machine from base

- Load machine onto lorry chassis

- Move lorry near main entrance

- Hoist down machine to the ground

- Insert heavy duty rollers to machine base and secured with bolts and nuts

- Push and position machine into the designated location inside workshop

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175