# METHOD OF STATEMENT

Date: 19th Oct 2020

Description of work

To unload 7 cases of 1 sets machine from 1 x low loader and 3 x 40ft HQ container, uncrate, shift and position into factory premise at ground level

Machine Spec

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

24/10/2020 (Saturday)

Time of Operation

09:45 am

Location of Operation

#### No 8 Tuas Ave 20

Avitron Pte Ltd

Equipment Use

1 x 70 ton mobile crane

Lorry loader

Heavy duty rollers

Wire/webbing sling

1 x 4.5 ton forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet )                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated hoisting area for unloading of machine

- Hoist up machine case from FR container and load onto ground

- Uncrate case

- After uncrate, hoist up and detach machine from base

- Swing machine away from base

- Lower down machine slowly to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175