Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 92 Tuas South Ave 3 | Lorry Crane Operation | 1 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 2 | Setting up of crane on site |  |
|  |  |  | 3 | Lifting operation |  |
|  |  |  | 4 | Using 4.5ton forklift to support in upright tank |  |
|  |  |  | 5 | Operating of boom lift inside plant |  |
|  |  |  | 6 | Working at height to attached sling onto tank |  |
|  |  |  | 7 | Signalling of load to drop off location |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lorry Crane Operation | RA Member: Zailani Supervisor |  |  |  |
| Location: | 92 Tuas South Ave 3 PUB | RA Member: Daniel Wong Manager |  |  |  |
|  |  | RA Member: Wong Yen Siong Site/Lifting Supervisor |  |  |  |
|  |  | RA Member: Asfandi Rigger/Signalman |  |  |  |
|  |  | RA Member: Ahmad Rigger/Signalman |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/06/2020 |  | Designation: | Director |  |
| Next Review Date: | 30/05/2023 |  | Date: | 20/07/2020 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premise | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 3 | 2 | 6 | Vehicle must not be left unattended while engine is running | 3 | 1 | 3 | Supervisor | 20-24 /07/20 |  |
| 2 | Setting up of crane | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction | 4 | 2 | 8 | Ensure no unauthorised personnel or vehicle near moving parts of crane Deployment of signalman to guide the crane operator of blind spots | 4 | 1 | 4 | Crane Operator | 20-24 /07/20 |  |
|  |  | Crane topple due to outriggers not fully extended | Bodily Injury & Damage to Property | - Site survey and visual assessment are to be made by competent person to ensure the correct lifting equipment is use for the lift - Crane operator to ensure all outrigger beams is fully extended and resting on steel plate as base plate - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Crane operator to ensure crane is able to lift weight of load at required radius - PPE (safety shoe, safety helmet) provided and put on | 4 | 2 | 8 | Lifting plan is to be briefed to all lifting personnel on site Barricade off the work area | 4 | 1 | 4 | Crane Operator | 20-24 /07/20 |  |
| 3 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment use are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | All lifting work are to be carried out as directed in the lifting assessment Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation | 4 | 1 | 4 | Lifting Supervisor | 20-24 /07/20 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lift | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting Supervisor | 20-24 /07/20 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent dislodgment - Ensure load is rig at it centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting Supervisor | 20-24 /07/20 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - To coordinate well with the crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all workers are hand off any loose parts of sling | 3 | 1 | 3 | Lifting Supervisor | 20-24 /07/20 |  |
| 4 | Using 4.5 ton forklift to support in upright tank | Failing of lifting equipment | Bodily Injury & Damage to Property | - Riggers to ensure lifting gears use to attached tank onto forklift are certified and has valid certificate - Only certified forklift operator to operate the forklift - Forklift operator to standby to reverse when lorry crane is slowly hoisting up to upright tank to prevent from swinging - To keep working path clear with no objects and unauthorised personnel obstructing the work area | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Lifting Supervisor on site | 20-24 /07/20 |  |
| 5 | Operating of boom lift inside plant | Hitting people or property | Bodily Injury & Damage to Property | -Only certified and competent boom lift operator are allowed to operate the boom lift - Operator advised when moving forward or reversing - To clear al material or tooling that is obstructing the work area | 4 | 2 | 8 | Slow operation speed | 4 | 1 | 4 | Supervisor on site | 20-24 /07/20 |  |
| 6 | Working at height to attached sling onto tank | Fall from height | Bodily Injury | - Provision of boom lift as working platform to work at height - Only certified and competent boom lift operator are allowed to operate the boom lift - Workers working at height on top boom lift to put on proper PPE (safety helmet, safety harness - Ensure safety harness use is in good condition, properly strapped on and hook to strong anchorage point | 4 | 2 | 8 | All trained and authorised operator record to be paste at the boom lift | 4 | 1 | 4 | Lifting Supervisor on site | 20-24 /07/20 |  |
| 7 | Signalling of load | Load hit structure due to improper signalling | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor on site - Only qualified signalman to execute the operation - Use of walkie-talkie to communicate whenever signalman is not visible to the crane operator | 4 | 2 | 4 | Close supervision by supervisor | 4 | 1 | 4 | Lifting Supervisor | 20-24 /07/20 |  |
|  |  | Load swing | Bodily Injury & Damage to Property | - Tagline should be used to control load from swinging so as to prevent bodily contact with load - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 4 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting Supervisor | 20-24 /07/20 |  |