Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 22 Pioneer Crescent | Lorry loader operation | 1 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 2 | Setting up of crane on site |  |
|  |  |  | 3 | Lifting operation |  |
|  |  |  | 4 | Signalling of load to location |  |
|  |  |  | 5 | Working at height on top platform |  |
|  |  |  | 6 | Adverse weather condition |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lorry Loader Operation | RA Member: Zailani |  |  |  |
| Location: | 22 Pioneer Crescent Dyson | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date: | 30/06/2023 |  | Date: | 09/09/2020 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premise | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with the occupier for safe route access - To clear all obstruction in way of moving path - Observe speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle to be moved into site with the aid of banks man guiding the driver to location | 4 | 1 | 4 | Supervisor on site | TBA |  |
| 2 | Setting up of crane on site | Crane topple due to outrigger not fully extended | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator to check and ensure that the crane is in good working condition - All outrigger beams is fully extended and resting on steel plate/timber mats as base plate - Crane operator to be on site when extending or retracting outrigger (must be able to see all operation procedure) - Lifting supervisor to verify and ensure that outrigger is fully extended before any commencing work is done | 4 | 2 | 8 | Ensure no unauthorised personnel or vehicle near moving parts of crane Competent person to check on the ground conditions prior to deployment of outriggers | 4 | 1 | 4 | Lifting Supervisor on site | TBA |  |
|  |  | Lifting area not cordon off | Bodily Injury | - To demarcate off lifting area with safety cones, red/white tape to ensure no entry to unauthorised personnel - Execute operation during non-peak hours if possible - Total closure of parking/ hoisting area when required | 4 | 2 | 8 | Assignment of traffic controller to guide and control movement of traffic and personnel within the vicinity | 4 | 1 | 4 | Supervisor on site | TBA |  |
| 3 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk or start/stop the load Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation in accordance to the lifting plan and SWP | 4 | 1 | 4 | Lifting Supervisor on site | TBA |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lift | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting Supervisor on site | TBA |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew, such as lifting supervisor, rigger and signalman is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent dislodgment - Ensure machine is rig at it centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only To provide crane hook with safety latch to prevent displacement of sling or load from the hook | 4 | 1 | 4 | Lifting Supervisor on site | TBA |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - To coordinate well with crane operator - Use glove | 3 | 2 | 6 | Supervisor to ensure all workers are hand off any loose parts of sling | 3 | 1 | 3 | Lifting Supervisor on site | TBA |  |
| 4 | Signalling of load to location | Load hit structure due to improper signalling | Bodily Injury & Damage to Property | - Only qualified signalman to execute the operation - Tagline should be used to control load from swinging - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - Use of walkie-talkie to communicate whenever signalman is not visible to crane operator - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Close supervision by lifting supervisor | 4 | 1 | 4 | Lifting Supervisor on site | TBA |  |
| 5 | Working at height on top platform | Falling from height | Bodily Injury | - Supervisor to ensure workers working at height on top platform to put on safety harness, safety helmet - Self Retracting Lifeline provided and properly secured onto building pillar to act as anchorage point - Ensure safety harness are properly strapped on and hook onto Self Retracting Lifeline while working on top platform - To ensure fall prevention equipment are adequate for all personnel working on top platform - Supervisor to ensure removable side railing of platform must be in place - Practice good housekeeping | 4 | 2 | 8 | Full supervision and regular remainder by supervisor and to ensure all safety precaution are in place Check all PPE, including safety harness and SRL are in good working condition and free from defects | 4 | 1 | 4 | Supervisor on site | TBA |  |
|  |  | Platform move or sway | Bodily Injury & Damage to Property | - Ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab - Ensure platform is stabilise effectively - Use strong fastening belt to secured front left and right of platform to building railing/pillar to prevent platform swing outward | 4 | 2 | 8 | Close supervision by site supervisor and ensure platform is fully stabilise | 4 | 1 | 4 | Supervisor on site | TBA |  |
|  |  | Falling tool | Bodily Injury & Damage to Property | - Ensure all loose tools are keep or removed immediately after use - Keep tools inside tool boxes - Never throw tools around after use - Barricade hazard area below platform and post warning sign to warn nearby personnel | 4 | 2 | 8 | Use of tool lanyards if require, to prevent of falling tool on below people | 4 | 1 | 4 | Supervisor on site | TBA |  |
| 6 | Adverse weather condition | Poor visibility due to heavy rain / lightning strike | Cause unexpected danger in term of workers slipping/strike by lightning | - To stop work immediately if there is heavy rain and lightning in the area - To evacuate to nearest shelter if there is lightning present | 4 | 2 | 8 | Supervisor to ensure all machineries are turned off and guide workers to the nearest shelter | 4 | 1 | 4 | Supervisor on site | TBA |  |