Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 750A Chai Chee Rd | Mobile/lorry crane operation | 1 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 2 | Setting up of crane on site |  |
|  |  |  | 3 | Lifting operation |  |
|  |  |  | 4 | Signalling of load to location |  |
|  |  |  | 5 | Working at height on top platform |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Mobile/Lorry Crane Operation | RA Member: Zailani |  |  |  |
| Location: | 750A Chai Chee Rd #04-02/03 Cam Plas (S) Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date: | 30/06/2026 |  | Date: | 29/08/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premise | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with occupier for safe route access - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to drive at slow speed and observed speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 18/09/25 |  |
| 2 | Setting up of crane on site | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Crane operator to ensure all outrigger beams is fully extended and resting on steel plate/timber mats as base plate - To determine the load radius and slewing part of the crane to ensure no hitting of nearby building or equipment - PPE (safety shoe, safety helmet) provided and put on | 5 | 2 | 10 | Ensure no unauthorised personnel or vehicle near moving parts of crane Barricade off the work area Signage to be up at the barricade area Assignment of traffic controller to control movement of vehicle and people within the vicinity | 5 | 1 | 5 | Lifting Supervisor on site | 18/09/25 |  |
| 3 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for mobile/ lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | While moving or slewing the crane, do it smoothly without sudden acceleration or deceleration Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation No lifting is to be done during bad weather | 5 | 1 | 5 | Lifting Supervisor on site | 18/09/25 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted - Implement of Lifting Plan to all lifting personnel on site to ensure that lifting operation is properly manage | 5 | 2 | 10 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 5 | 1 | 5 | Lifting Supervisor on site | 18/09/25 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent dislodgment - Ensure machine is rig at it centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 5 | 2 | 10 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only To provide crane hook with safety latch to prevent displacement of sling or load from the hook | 5 | 1 | 5 | Lifting Supervisor on site | 18/09/25 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and to ensure all workers are hand off any loose parts of sling | 3 | 1 | 3 | Lifting Supervisor on site | 18/09/25 |  |
|  |  | Load lifting device gave way | Bodily Injury & Damage to Property | - To ensure bolts connecting lifting device to the machine is properly secured and tighten using spanner or allen key - Ensure bolts specification are comparably to the machine track - To inform supervisor of any loose bolts which is incomparable to the machine track | 5 | 2 | 10 | Eric Wong supervisor to view the lifting device together with machine manufacture personnel on site before any hoisting work | 5 | 1 | 5 | Lifting Supervisor on site | 18/09/25 |  |
| 4 | Signalling of load to location | Load hit structure due to improper signalling | Bodily Injury & Damage to Property | - Only qualified signalman to execute the operation - Tagline should be used to control load from swinging so as to prevent bodily contact with load - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - Signalman to position himself to be visible to crane operator through hand signalling method - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | Use of walkie-talkie to communicate whenever signalman is not visible to crane operator Close supervision by lifting supervisor | 5 | 1 | 5 | Lifting Supervisor on site | 18/09/25 |  |
| 5 | Working at height on top platform | Falling from height | Bodily Injury | - Workers working at height on top platform are adequately trained and have attended WAH course - Self Retracting Lifeline provided and properly secured onto building railing/pillar to act as anchorage point - Supervisor to ensure workers working at height on top platform is fully equip with safety helmet, safety harness - Ensure safety harness are properly strap on and hook onto Self Retracting Lifeline while working on top of platform at all time - To ensure fall prevention equipment are adequate for all personnel working on top platform - Supervisor to ensure removable side railing of platform must be in place - Practice good housekeeping | 5 | 2 | 10 | Full supervision and regular remainder by WAH manager and to ensure all safety precaution are to be followed and in place | 5 | 1 | 5 | Lifting Supervisor on site | 18/09/25 |  |
|  |  | Platform move or sway | Bodily Injury & Damage to Property | - Ensure platform 3ft metal plate protruding is fully lean and rested on the opening slab - Ensure platform is stabilise effectively - Use strong fastening belt to secured front left and right of platform onto building railing/pillar to prevent platform swing outward - Ensure platform is tied back to the competent point of platform using lorry crane | 5 | 2 | 10 | Close supervision by site supervisor and ensure platform is fully stabilise before proceeding to work on top platform | 5 | 1 | 5 | Lifting Supervisor on site | 18/09/25 |  |